SEW YOUR OWN: SNOOD

Select the fabric you wish to use first.
Ideally, use stretch fabric for your snood. 
If you plan to use a non-stretch material, make your snood another 3 inches longer.
*

- Cut 2 lengths – one main fabric (outer) piece and one inner piece:

Length: 70cm / 28” inches, Width: 28cm / 11” inches

STRETCH OF FABRIC

- Place fabric with right sides together (patterned side facing into the centre/back of fabric facing up at you) and pin.

- Once pinned, stitch the long edges together (top and bottom) only.

- You now have a tube! From here, fold the fabric back on itself, so the short sides are facing right sides together (see example below – copyright SewHayleyJane).

- Pin the tube, leaving enough space for a turning gap (around 4-5cm)

- Now sew all the way around, using a backstitch at the beginning and ending – remember your turning gap (red marks) – do not sew this area!

- Once done, start to pull your fabric through the hole – this will turn your project the right way around!

Ta-daaa! It should look like a lovely Snood now!

- There will be a small hole where your turning gap is still visible – there are 2 ways to close this gap, depending on how tidy you wish the project to be:

	1) Pull the fabric of the hole together, and sew with your machine – try to match the fabric colour so it is less obvious
	2) Use an invisible/ladder stitch. This is a well-known hand-stitch which seamlessly covers up holes (great for holes in clothes too!). 
I will help you in the classroom, but for help at home, see YouTube: https://youtu.be/K3fFHV6r9Ow

Thank you & enjoy your snood!